# Changes to the avoided costs used in the E3 Calculator for 2009 to 2011 energy efficiency program planning

The electricity avoided costs were updated to be consistent with the 2007 MPR inputs and methodology.  The update was directed by the Commission’s April 21, 2008 ruling, ASSIGNED COMMISSIONER’S AND ADMINISTRATIVE LAW JUDGE’S RULING REGARDING MAY 15, 2008 ENERGY EFFICIENCY PORTFOLIO PLANS FOR 2009-2011. (http://docs.cpuc.ca.gov/efile/RULINGS/81727.pdf)

The changes were as follows:

## Changes to the cost characteristics of the combined cycle gas turbine used to set the long-run annual average avoided cost of electricity

- The heat rate was changed to 6,946 Btu/kWh

- The capacity factor was reduced to 76%

- The capital cost was increased to $1,054/kW

- Fixed O&M was increased to $14.27/kW-Yr

- Variable O&M was changed to $2.53/MWh

- Financing assumptions were changed to 50% Debt with a 7.7% cost of debt and 13.3% cost of equity.

## Changes to the natural gas fuel cost forecast used to set the long-run annual average avoided cost of electricity

Consistent with the MPR methodology, and consistent with the 2006 avoided cost update for energy efficiency, E3 updated the near-term gas price forecast using the NYMEX cost of natural gas Delivered to Generator.  The near-term forecast (2009 through 2013) was based on 30-day Average ending April 21 2008.  E3 used the values at the NYMEX close for Natural Gas Futures, Socal Basis Swap and PG&E Citygate Basis Swap.   For contracts from January -April 2008, E3 used the closing price from final day of trading for each contract.

Over the next three years (2014 through 2016) the forecast transitions to the 2007 long-term MPR forecast, which is then used from 2017 through 2039.  The MPR model uses NYMEX Basis quote averages to calculate a single statewide average California border price.  For the avoided cost update, the same basis quotes that are used for the near-term forecast are used to calculate separate citygate price forecasts for each IOU.

## CO2 Cost Scenario

The updated planning calculators also include a CO2 cost scenario that replaces the 2006 adopted cost of CO2 with $30/tonne.  That value remains constant in nominal terms over all years in the scenario.

# Changes to the discount rates used in the E3 Calculators

As part of the updates, the discount rates were also updated to be consistent with D.07-12-049.  Per the direction of Energy Division, the pre-tax weighted average cost of capital for each investor-owned utility was used for the update.

The updated values are:

SCE		8.75%

SDG&E	8.40%

PG&E		8.79%

SoCal Gas	8.68%

SoCal Gas’s rate was not specified in that decision.  The updated value is based on the following: