# Cost Effectiveness Evaluation “Calculator” Tool v. 5

# Quick Guide and Equation Reference (v.5)

This document provides a brief guide to the user inputs and outputs.  This is followed by a detailed presentation of the equations used by the Calculator Tool.  The last section presents a discussion of the data management issues and steps taken in developing the inputs for the tool.

Table of Contents

Cost Effectiveness Evaluation “Calculator” Tool v. 3c2	1

Quick Guide and Equation Reference (v.3c2)	1

Summary of Revisions	3

Inputs Summary	3

Output	8

Notes on the Tests and Outputs	8

Outputs	9

Output by Measure	17

Batch Processing	19

Processing Choice	19

Input File Rows	20

Control Tab	21

Equations	22

Net July-Sept Peak (CP_kW_N_TOT[Smr])	22

Net Dec-Feb Peak (CP_kW_N_TOT[Wtr])	23

Noncoincident Peak (Net NCP)	24

Noncoincident Peak by Year (NNCP[t])	24

CEC Peak (Net CEC)	25

Annual Net kWh (Annual_kWh)	25

Lifecycle Net kWh (Lifecycle_kWh)	25

Annual Net Therms (Annual_Therms)	25

Lifecycle Net Therms (Lifecycle_Therms)	25

Cost Effectiveness (Lifecycle Present Value Dollars)	26

Cost	26

TRC Cost (TOTCOSTTRC)	26

PAC Cost (TotCostPAC)	27

RIM Cost (TotCostRIM)	27

Lifecycle Benefit	28

Electric TRC Benefits (NetPVBenTOT[E])	28

Gas TRC Benefits (NetPVBenTOT[G])	30

Levelized Cost and Benefit	32

Discounted Electric Reductions (kWhD[N])	32

Discounted Gas Reductions (ThD[N])	33

Levelized Cost	33

Levelized Benefits	34

Levelized Benefit – Cost	34

Emissions Reductions	36

Electric Reductions:  CO2 tons per year (Emission[E][CO2])	36

Gas Reductions:  CO2 tons per year (Emission[G][CO2])	36

Monthly Impacts (IMP_MO[X], IMP_MO[Th])	37

Sector and End Use Shape Combinations	38

# Summary of Revisions

Version 3c

- Rebate entries modified to be more explicit between transfer payments and administrative costs.

- Net to gross ratios can now be entered directly in Column I.  Previously the user needed to select a Program Type and the Calculator would use NTG values that were preloaded in a lookup table.

- Output descriptions updated to reflect measures installed in the year, rather than measures in place during a specific quarter of the year.

Version 4a

- Revise calculation of TRC to comply with Shareholder Incentive Decision, ordering paragraph 14.  TRC Cost = Admin$ + NTG*Meas$ + (1-NTG) * (Rebate$ + OtherInc$ + DirectI$)

Version 5

- Modify to allow use for 2009-2011 program planning

  - Change first year of implementations to 2009

  - Change presnt value calculations to discount to the beginning of 2009

  - Change PG&E and SDG&E calculators to use annual installations

# Inputs Summary

| Item | Location | Comment |
| --- | --- | --- |
| Proposer Name | Cell D4 | Text field |
| Program Name | Cell D5 | Text field |
| Contact Information | D9:D13 | Text fields |
| Market Sectors | E14:E15 | Text fields. Only used for PG&E reporting. Do not affect calculations. |
| Program Budget |  |  |
| Administrative Costs: Overhead and G&A | G3 | Total cost over the program implementation cycle. Entered in nominal dollars |
| Administrative Costs: Other Admin Costs | G4 | Total cost over the program implementation cycle. Entered in nominal dollars |
| Market/Outreach | G5 | Total cost over the program implementation cycle. Entered in nominal dollars |
| Direct Implementation: Activity Installation Hardware & materials Rebate processing and inspection | G7 to G10 | Total cost over the program implementation cycle. Entered in nominal dollars. These cell entries are for non-incentive related costs. Incentive and rebate information is entered on a per-measure basis or in cell I4. |
| Total Incentives and Rebates | G11 | Not a user entry. Sum of values in Cells I4 to I8 |
| EM&V | G12 | Entered in nominal dollars. |
| Costs recovered from other sources. | I12 | Entered in nominal dollars. |
| User Input Incentive | I4 | Entered in nominal dollars. Use this cell for incentives and rebates that are not calculated on a per unit installed basis. Users should be careful to avoid double counting incentives. If this cell is used, only enter incentive values in columns M through P for measures that are not included in this incentive calculation. |
| Rebate | I5 to J5 | Not a user entry. Rebate to end use customer or its assignee. Nominal and present value totals based on rebates per unit entered in column M. Note that the value shown in column I is in nominal dollars, and column J is a present value (to reflect the expected timing of the payments). Nominal values are used for program budget reporting and PV is used for the cost effectiveness and levelized cost calculations. |
| Direct Install Labor | I6 to J6 | Not a user entry. Nominal and present value totals based on rebates per unit entered in column O. See above for further discussion of nominal and PV dollars. |
| Direct Install Materials | I7 to J7 | Not a user entry. Nominal and present value totals based on rebates per unit entered in column P. See above for further discussion of nominal and PV dollars. |
| Upstream Payments | I8 to J8 | Not a user entry. Nominal and present value totals based on rebates per unit entered in column N. See above for further discussion of nominal and PV dollars. |
| Program Inputs (Inputs begin in Row 17) |  |  |
| Program Name | Col A | Not a user entry. Hidden text field used in reporting information to the Energy Division. |
| Measure Name | Col B | Text field |
| DEER RunID | Col C | Numeric field corresponding to DEER database. |
| Climate zone | Col D | Select from drop-down list. Cannot be left blank. Used to determine the impacts and costs for the measure. ‘System’ applies the generation and T&D avoided costs from the climate zone with the lowest average generation and transmission avoided capacity cost over 25 years. Note that Climate zone 3 has been subdivided into two subzones. Climate zone 3A is for San Francisco and the Peninsula. Climate Zone 3B is for the East Bay. See the attachment for a complete listing of Cities and Climate Zones. |
| Target Sector | Col E | Select from the Drop Down list. This entry is used to determine whether to use a residential or non-residential TOU correction factor where applicable. For the PG&E, SCE and SoCalGas tools, this entry is also used to limit the measure end use shapes listed in the drop down entries in Column F. This entry is not used to constrain measure end use shapes in the SDG&E model, although it is used for reporting purposes. |
| Measure End Use Shape | Col F | Select from the drop-down list. Note that the list of measures can vary depending upon the Target Sector selected in Column E. For PG&E, SCE, and SoCal Gas, this entry will determine if a measure could receive a TOU correction factor adjustment. Having one of the following end use shapes is a necessary, but not sufficient, condition to qualify for the TOU correction adder. In order to receive a TOU correction factor adjustment, a measure must also have a positive entry in column Z % Eligible for TOU AC Adjustment. PG&E: 26 = Res. Central Air Conditioning 22 = Res. Ht. Pump Cooling 29 = Res. New Const. Cooling 33 = Res. Insul. Cen. A/C 35 = Res. Ceil. Insul. HP Cooling 37 = Res. Wall Insul. HP Cooling 39 = Res. Flr. Insul. HP Cooling 45 = Res. Dir. Assist. Evap. Cooler 3 = Commercial HVAC SCE: AC-NC New_AC-Ret; AC_Cooling-RC HeatPump_ThroughWall-RC Package_AC-NC <65K_AC_Split-NC <65K_AC_Pckg-NC 65K-135K_Air_AC-NC 65K-135_Wtr_AC-NC >135K_Air_AC-NC >135K_Wtr_AC-NC <65K_EvapCool-NC Evap_Cooling-Ret New_HtPmp-Ret Replace_Chiller-Ret Wtr_Cool_Chiller-NC SoCal Gas: AC_Cooling AC >135K_Air_AC >135K_Wtr_AC New_AC New_HtPmp Reduce_Cooling_Load |
| CZ, Sector, Measure combination found? | Col G | Not a user input. If FALSE, then recheck the measure end use shape drop down list to verify that you have selected a valid measure. Typically, the FALSE warning will appear when the user selects a measure based on a certain Sector, but then changes the Sector without updating the measure choice. Note: This column is not used in SDG&E’s tool. |
| Measure Type | Col H | Select from drop-down list. This entry is used to determine the Measure Life, based on values in the Program Manual. Users may also specify a life using the “Generic Life” entries at the bottom of the drop down list. |
| Program Type | Col I | Select from drop-down list. This entry is used to determine the Net-to-Gross ratio, based on values in the Program Manual. Version 3c modification: users can now directly enter numeric values in this column. |
| Unit Definition | Col J | Not used by the spreadsheet. For informational purposes only. |
| Program Type | Col K | Currently not used. Intent is to indicate if a measure is used in a new construction (NEW), replace-on-burnout (ROB), or early replacement (retrofit (RET)) application to refine cost-effectiveness analyses in the future. |
| Gross Measure Cost | Col L | For NEW and ROB applications, this is the cost of the efficient equipment less the cost of the standard efficiency equipment. Use value in DEER as applicable (adjusted for inflation as needed). Include initial capital costs, including sales tax, ongoing O&M costs including fuel, removal costs less salvage value, and value of customers time in arranging for installation (if significant). For RET applications, this if the full cost of the efficient equipment plus installation costs. For direct install programs, enter the sum of Direct Install Labor and Direct Install Materials (Col O + Col P) Note that the E3 Calculator does not inflate any of the costs in columns L through Q. If the nominal cost of the measure is expected to change over the implementation period, the user can enter an adjusted cost value such that the present value of the adjusted cost is the same as the present value of the expected annually varying costs. |
| Rebate to end use customer or its assignee | Col M | Per unit dollar rebates paid to the participant or its assignee by the sponsoring agency. |
| Incentives to entities other than the end use customer or its assignee | Col N | Per unit incentives. Also referred to as upstream incentives. |
| Direct Install Labor | Col O | Per unit labor provided to the participant by the proposed Program. |
| Financial Incentive: Direct Install Materials | Col P | Per unit materials provided to the participant by the proposed Program. |
| Gross Participant Cost | Col Q | Not a user entry. Equals the Gross measure cost less the incentives in columns M, N, O, and P. . This treatment corrects the application of the net-to-gross ratio to conform to the ALJ ruling in the Shareholder incentive proceeding. |
| Gross Unit Annual Electricity Savings | Col R | Annual net kWh reduction attributable to the installation of one unit of the measure. If a measure has an EUL less than one year, the annual net kWh reduction should reflect the savings assuming that the device were in place for a full year. |
| Electric Rate Schedule | Col S | Select from drop-down list. Used to determine revenue loss for RIM test. |
| Demand Scale Factor | Col T | Not a user input. Indicates “kWh” if capacity savings are calculated using annual kWh reductions, or “kW” if capacity savings are calculated using summer peak kW reductions. |
| User entered kW savings per unit | Col U | Summer peak period kW reduction attributable to installation one unit of the measure. Used for reporting kW reductions. Also used for cost effectiveness calculations if Col T indicates “kW.” Otherwise, the cell is grayed out, indicating that the value is not used for the cost-effectiveness calculations. |
| Gross Unit Annual Gas Savings | Col V | Annual net therms reduction attributable to the installation of one unit of the measure. |
| Gas Sector | Col W | Select from the drop-down list. Used to determine the gas avoided costs to apply to the measure. Note: This column is not a user input in PG&E’s residential tool. |
| Gas Rate Schedule | Col X | Select from drop-down list. Used to determine revenue loss for RIM test. Note: This column is not a user input in PG&E’s residential tool. |
| Gas Savings Profile | Col Y | Select from the drop-down list. Used to determine the gas avoided costs to apply to the measure. - Summer Only: All gas savings occur in April through September. - Winter Only: All gas savings occur in October through March - Annual: Gas savings occur uniformly throughout the year. |
| Placeholders, not currently used | Col Z to AC | Currently not used. Intent is to combine this information with indications if a measure is used in a new construction (NEW), replace-on-burnout (ROB), or early replacement (retrofit (RET)) application to refine cost-effectiveness analyses in the future. |
| Combustion Type | Col AD | Determines the rate of emission savings per MMBTU of reduced natural gas usage from gas conservation. (lbs of CO2, NOX, and PM-10). Not a user input for PG&E. Not used for SCE. Select from a drop down list for SDG&E and SoCalGas. Res Furnace is used as a default if there is no user selection. |
| Effective Useful Life | Col AE | For PG&E and SCE, determined by the spreadsheet, based on Measure Type. For SDG&E and SoCal Gas, the numeric value is entered directly in this column. |
| Net –to-Gross Ratio | Col AF | Determined by the spreadsheet, based on Program Type or the numeric user entry in column I |
| % Eligible for TOU AC Adjustment | Col AG | User input between 0 and 100%, with 100% for those cases where the entire measure receives the AC TOU correction factor. Note that the measure will not receive a correction factor adjustment for PG&E, SCE, or SoCalGas if the measure end use shape does not qualify (see discussion for Col F above). |
| Installation Schedule |  |  |
| Installations by Year | Col AH to Col AN | The annual installations are assumed to occur uniformly over the 4 quarters in each year. |
| Installations by Quarter | Col AP to Col BQ. | Not a user entry |
| Total Number of Units | Col BR | Not a user entry. |
| Comments | Col BS | For informational purposes only. Not used in the calculations. |

# Output

## Notes on the Tests and Outputs

#### Total Resource Cost Test (With externalities)

| Item | Comment |
| --- | --- |
| Tax Credits (TC) | No explicit input for tax credits. |
| Levelized benefits per kWh | Uses Net discounted kWh to correspond to Net benefits (NTG ratio adjusted) |

## Outputs

| Item | Location | Comment |
| --- | --- | --- |
| Total Program Budget | D4 to E4 | Total of all program budget items, including rebates and incentives. D4 is total nominal dollars, E4 is the NPV. Note that only the per unit costs are present valued. The lump sum admin costs are not adjusted unless a value less than 1.0 is entered in cell J1 of the INPUT tab. |
| Rebates (Transfers) | D5 to E5 | Total rebates to end users or their assignees. These are already included in the Total Program Budget in D4 or E4, as well as the Net participant cost (not reduced for rebates) shown in Cells D6 and E6. The Rebate costs are shown separately so that they can be removed from the TRC test to prevent double counting. Column D is nominal, column E is present value. |
| Net Participant Cost | D6 to E6 | Present value of participant costs before any reductions for rebates (transfers), multiplied by Net to Gross Ratio. Column D is nominal, column E is present value. |
| Program Impacts |  |  |
| Annual Net kWh | C10:C11 | Sum across all measures of their annual kWh savings adjusted for their net-to-gross ratio. Based on total units installed. For those measures with useful lives less than one year, the Annual Net kWh is multiplied by the EUL (See Annual_kWh) If a program has units that have short useful lives such that they “fail” prior to the end of the implementation period, the maximum annual kWh savings actually attained in any year will be lower than what is shown here. In addition, this metric does not reduce annual kWh for measures that are installed after the 1st quarter in a year. |
| Lifecycle Net kWh | D10:D11 | Sum across all measures of their lifecycle kWh savings, adjusted for their net-to-gross ratios. Lifecycle savings = Annual kWh * Measure Life (in yrs) * Net-to-gross ratio.* Total units installed. (See Lifecycle_kWh). |
| Annual Net Therms | E10-E11 | Sum across all measures of their annual therm savings adjusted for their net-to-gross ratio. . For those measures with useful lives less than one year, the Annual Net Therms is multiplied by the EUL (See Annual_Therms) For SCE, this metric is based on units installed and still effective in the quarters identified for Net July-Sept Peak. For SoCalGas and PG&E, this metric is based on total units installed, regardless of useful life. |
| Lifecycle Net Therms | F10:F11 | Sum across all measures of their lifecycle therm savings, adjusted for their net-to-gross ratios. Lifecycle savings = Annual therms * Measure Life (in yrs) * Net-to-gross ratio.* Total units installed. (See Lifecycle_Therms). Note: This metric is NOT based on the units in place during the third quarter of year 4.therms. |
| Net July-Sept Peak (kW) | G10:G11 | Sum across all measures of their average July through September coincident peaks. Based on measures installed in the calendar year (even if the measure is not installed until after September) Coincident peak is defined as the load during the five highest system load hours in each month. (See Equation CP_KW_N_TOT[Smr]). Value is adjusted for the net to gross ratio. |
| Net Dec-Feb Pk (kW) | H10:H11 | Sum across all measures of their average December, January, and February coincident peaks. Based on measures installed in the calendar year. Coincident peak is defined as the load during the five highest system load hours in each month. (See Equation CP_KW_N_TOT[Wtr]). Value is adjusted for the net to gross ratio. |
| Net CEC (kW) | I10:I11 | Annual Net kWh * 0.217 /1000. (metric used in prior filings) |
| User Entered kW | J10:J11 | Sum across all measures installed in the corresponding years, multiplied by their user entered peak grid kW savings (INPUT column U) adjusted for their net-to-gross ratio. |
| Cost Effectiveness |  | Lifecycle Present Value Dollars |
| Program TRC |  |  |
| TRC Cost | C16 | Total Program Budget + Net participant cost – Rebates. (See TOTCostTRC) |
| TRC Benefits: Electric | D16 | Sum of net electric present value benefits for all measures installed over the three years. “Net” indicates the gross benefits are multiplied by the Net-to-Gross ratios. (See NetPVBenTOT[E]) |
| TRC Benefits: Gas | E16 | Sum of net gas present value benefits for all measures installed over the full implementation period. “Net” indicates the gross benefits are multiplied by the Net-to-Gross ratios. (See NetPVBenTOT[G]) |
| Benefit – Cost NPV | G16 | TRC Benefits Electric +TRC Benefits Gas – TRC Cost |
| B/C Ratio | H16 | (TRC Benefits Electric + TRC Benefits Gas) / TRC Cost. Note that the B/C Ratio is an approximation because any supply costs associated with increased are treated as negative benefits rather than as a cost as in the Standard Practice Manual |
| PAC (Program Administrator Cost Test) |  |  |
| PAC Cost | C17 | Total program budget. (See TOTCostPAC) |
| PAC Benefits:Electric | D17 | Same as TRC test |
| PAC Benefits: Gas | E17 | Same as TRC test |
| Incentives | F17 | NA. Already included in PAC Cost above |
| PAC Benefit - Cost NPV | G17 | PAC Benefits Electric +PAC Benefits Gas – PAC Cost |
| PAC B/C Ratio | H17 | (PAC Benefits Electric + PAC Benefits Gas) / PAC Cost. Note that the B/C Ratio is an approximation because any costs associated with increased usage are treated as negative benefits rather than as a cost as in the Standard Practice Manual |
| RIM (Ratepayer Impact Measure Test) |  |  |
| RIM Cost | C18 | Total bill reductions plus Total Program Budget(See TOTCostRIM) |
| RIM Benefits:Electric | D18 | Same as TRC test |
| RIM Benefits: Gas | E18 | Same as TRC test |
| RIM Benefit - Cost NPV | G18 | RIM Benefits Electric + RIM Benefits Gas – RIM Cost |
| RIM B/C Ratio | H18 | (RIM Benefits Electric + RIM Benefits Gas) / RIM Cost. Note that the B/C Ratio is an approximation because any costs associated with increased usage are treated as negative benefits rather than as a cost as in the Standard Practice Manual |
| Levelized Cost and Benefit (discounted present values) |  |  |
| Discounted kWh (all tests) | C25:C27 | Discounted lifecycle avoided kWh, adjusted for Net-to-gross ratio for each measure. (See kWhD[N]) |
| Discounted Therms (all tests) | D29:D31 | Discounted lifecycle avoided therms, adjusted for Net-to-gross ratio for each measure. (See ThD[N]) |
| TRC Levelized Cost | E25, E29 | (TRC Benefits) less the (TRC Benefits – Costs) (See LC[TRC][E], LC[TRC][G]) |
| PAC Levelized Cost | E26, E30 | (PAC Benefits) less the (PAC Benefits – Costs) (See LC[PAC][E], LC[PAC[G]) |
| RIM Levelized Cost: Electric | E27 | RIM Cost * [RIM Electric Benefit /(RIM Electric Benefit + RIM Gas Benefit)] / Discounted kWh (See LC[RIM][E]) |
| RIM Levelized Cost: Gas | E31 | RIM Cost * [RIM Gas Benefit /(RIM Electric Benefit + RIM Gas Benefit)] / Discounted kWh (See LC[RIM][G]) |
| TRC Levelized Benefits | F25, F29 | TRC benefits (by fuel type) divided by the TRC discounted kWh or Therms. (See LB[TRC][E], LB[TRC][G]) |
| PAC Levelized Benefits | F26, F30 | PAC benefits (by fuel type) divided by the PAC discounted kWh or Therms. (See LB[TRC][E], LB[TRC][G]) |
| RIM Levelized Benefits | F27, F31 | RIM benefits (by fuel type) divided by the RIM discounted kWh or Therms. (See LB[TRC][E], LB[TRC][G]) |
| TRC Levelized Benefit - Cost | F25, F29 | Sum over all measures of TRC benefits less the allocated TRC costs for each measure. For each measure, unit costs (net IMC) are allocated between electric and gas in proportion to the benefits by fuel for that measure. The remaining program costs are allocated to each measure based on each measure’s share of the total electric and gas benefits. Once allocated to each measure, the remaining program costs are allocated to electric and gas in proportion to the benefits by fuel for that measure. (See LBC[TRC][E][WA], LBC[TRC][G][WA]) |
| PAC Levelized Benefit - Cost | F26, F30 | Sum over all measures of PAC benefits less the allocated PAC costs for each measure. For each measure, unit costs (per unit rebates, direct install labor, and direct install materials) are allocated between electric and gas in proportion to the benefits by fuel for that measure. The remaining program costs are allocated to each measure based on each measure’s share of the total electric and gas benefits. Once allocated to each measure, the remaining program costs are allocated to electric and gas in proportion to the benefits by fuel for that measure. (See LBC[PAC][E][WA], LBC[PAC][G][WA]) |
| RIM Levelized Benefit - Cost | F27, F31 | RIM Levelized Benefit – RIM Levelized Cost. (Also, see LBC[RIM][E][WA], LBC[RIM][G][WA]) |
| Emissions Reductions |  |  |
| Electric Reductions | C36:E43 | Units implemented in the year, times the annual emission reduction (due to reduced electricity consumption) for the measure, adjusted for the net-to-gross ratio. (See Emission[E][?]) |
| Gas Reductions | F36:G43 | Units implemented in the year, times the annual emission reduction (due to reduced gas consumption) for the measure, adjusted for the net-to-gross ratio. (See Emission[G][?]) |
| Lifecycle Electric Reductions | C46:E53 | Units implemented in the year, times the annual emission reduction (due to reduced electricity consumption) for the measure, times the expected useful life of the measure, adjusted for the net-to-gross ratio. (See Emission[E][?]) |
| Lifecycle Gas Reductions | F46:G53 | Units implemented in the year, times the annual emission reduction (due to reduced gas consumption) for the measure, times the expected useful life of the measure, adjusted for the net-to-gross ratio. (See Emission[G][?]) |
| Net Impacts Summary by Year |  |  |
| Annual Net kWh | C58:C65 | Sum of # of measures installed in the year * Annual kWh reduction per measure * Net-to-gross ratio |
| Lifecycle Net kWh | D58:D65 | Sum of # of measures installed in the year * Annual kWh reduction per measure * Net-to-gross ratio * Measure expected useful life |
| Annual Net Therms | E58:E65 | # of measures installed in the year * Annual Therm reduction per measure * Net-to-gross ratio |
| Lifecycle Net Therms | F58:F65 | Sum of # of measures installed in the year * Annual Therm reduction per measure * Net-to-gross ratio * Measure expected useful lifn. |
| Net July-Sept kW (OnPeak) | G58:G65 | Sum of # of measures installed in the year * Annual summer peak reduction per measure * Net-to-gross ratio(See IMP_MO[X]) |
| Net Dec-Feb (kW) | H58:H65 | Sum of # of measures installed in the year * Annual winter peak reduction per measure * Net-to-gross ratio Coincident peak is defined as the load during the five highest system load hours in each month. (See Equation CP_KW_N_TOT[Wtr]). Value is adjusted for the net to gross ratio. |
| Net CEC (kW) | I58:I65 | Annual Net kWh * 0.217 / 1000 |
| Net Impacts by Sector |  | Same information as the Net Impacts Summary section, except segmented by customer sector instead of year of installation. |
| Net Impacts by CPUC End Use Categories |  | Same information as the Net Impacts Summary section, except segmented by end use categories instead of year of installation. Note that after consultation with the IOUs, the categories were modified slightly. The detailed process categories were abandoned in favor of a single “Process”; and Domestic Hot Water was made a category distinct from Water Heating. |
| Net Impacts by Climate Zone |  | Same information as the Net Impacts Summary section, except segmented by location instead of year of installation. |
| Persistent reductions in the summer or winter | C145:D152 | Peak reductions tracked, not by the year of installation, but by the amount of peak load reduction that has been installed and not reached the end of its expected useful life in the 3rd of 4th quarter of each year. |
| Monthly Impacts |  | All impacts are based on the devices that would be in place in that month. All measures are assumed to be installed on the first say of the quarter (no ramp-up). (see IMP_MO[x]). Note that the NCP values will not match what is shown in the Program Input section As some measures will have maximum reductions in different months, the NCP reductions in any particular month will be lower than the annual NCP reduction (which uses the maximum reduction for each measure, regardless of when that reduction occurs) |

## Output by Measure

| Net Impacts by Measure | Cols B through I | Same information as the Impacts Summary section, except segmented by measure instead of year of installation. |
| --- | --- | --- |
| Levelized Benefits less Costs |  |  |
| w/ allocated Admin | Cols L,M,P,Q | Same information as in the Output Tab for levelized results, but reported by measure. |
| No Allocated Admin – TRC | Cols J,K | Only includes net participant costs and rebates and incentives that are entered on a per-measure basis in costs. Lump sum program costs and other lump-sum admin costs are not included. (See LBC[TRC][E][NA], LBC[TRC][G][NA]) |
| No Allocated Admin - PAC | Cols N,O | Includes per unit costs for 1) rebates, 2) direct install labor, 3) direct install materials, and 4) upstream payments. Other rebates and incentives (entered as a lump sum on the Input tab), as well as other program and admin costs are not included. (See LBC[PAC][E][NA], LBC[PAC][G][NA]) |
| Benefit Cost Ratios |  |  |
| TRC – No allocated admin | Col R | Measure present value net benefit / present value net cost. Cost is net participant cost and rebates and incentives that are entered on a per-measure basis. |
| PAC – No allocated admin | Col S | Measure present value net benefit / present value net cost. Cost is per-unit rebate and incentive costs. |
| TRC w/ allocated admin | Col T | Measure present value net benefit / present value net cost. Cost is net incremental measure cost plus allocated admin costs. |
| PAC w/ allocated admin | Col U | Measure present value net benefit / present value net cost. Cost is per-unit rebate and incentive costs plus allocated admin costs. |

# Batch Processing

A “Process Files” button is located on the INPUT and CONTROL tabs.  These buttons activate Excel Visual Basic macros that facilitate the separation of calculator input and outputs from the main calculation spreadsheet. (Note, details for the batch processing section may differ slightly for each IOU, as small modifications are ongoing to accommodate specfic IOU batch processing needs.)

Pressing the button calls up the following dialog box

### Processing Choice

Let me select one file to import.  Select this option to check the calculations for one file

Process steps are:

- Open a file selection dialog box for the user to select the file to import (Source file)

- Open the selected source file, and copy data from the INPUT tab of the input file

- Past the INPUT data into the Master Calculator file

- Close the source file

Let me select one or more files to process.  Select this option to batch process one or more source files that are in the same folder.  Batch process refers to importing data from the source file and exporting the results back to that same file.

Process steps are:

- Open a file selection dialog box for the user to select the files to import (Source files).  Use the shift key of the control key to select more than one file.

- Open the selected source file, and copy data from the INPUT tab of the input file

- Past the INPUT data into the Master Calculator file

- Calculate the program results for the imported data, and copy the results in the OUTPUT and OUTPUT BY  MEASURE tabs back to the source file.

- If the user has input a suffix (see Names for Processed Files on the dialog box above) then append this suffix to the end of the original source file name and save the combined input and output tabs as the new name.

- If the user have not entered a suffix, save the combined input and output tabs using the original name (overwrite the original file)

- Close the source file, and repeat starting at step 2.

Batch process the files listed on the Control tab.  Select this option to batch process one or more source files that reside in various folders.  Batch process refers to importing data from the source file and exporting the results back to that same file.

Process steps are:

- Open the selected source file, using the path and filename information in the CONTROL tab.

- Copy data from the INPUT tab of the input file

- Past the INPUT data into the Master Calculator file

- Calculate the program results for the imported data, and copy the results in the OUTPUT and OUTPUT BY  MEASURE tabs back to the source file.

- If the user has input a suffix (see Names for Processed Files on the dialog box above) then append this suffix to the end of the original source file name and save the combined input and output tabs as the new name.

- If the user have not entered a suffix, save the combined input and output tabs using the original name (overwrite the original file)

- Close the source file, and repeat starting at step 1.

Just export the current program.  Select this option to create a small  INPUT and OUTPUT file using the data currently in the Master Calculator.

Process steps are:

- Copy the INPUT data to a new worksheet

- Copy the OUTPUT and OUTPUT BY  MEASURE tabs to the new worksheet.

- Prompt the user to save the new worksheet with just the input and output data.

### Input File Rows

Select the maximum number of measures allowed in the source spreadsheet

# Control Tab

The control tab is used by the Process Files macro button for batch processing.  The user can enter directory path and filename information into this file for unattended batch processing of files in multiple directories.  Note that the default is to overwrite these source files with the processed INPUT/OUTPUT file unless a file suffix is specified in the batch processing dialog box.

We recommend that you make a duplicate of your files and directories and run the batch processor on these file to avoid accidental loss of the original files.

| Number of Files to Process | D11 | Number of files to process through the Calculator file. |
| --- | --- | --- |
| Directory Paths | C15:C94 | Directory path for the INPUT files to process |
| FileNames | D15:D94 | Name of the file to process. |
| Process Status | B15:B94 | Indicates whether the file is processed successfully |

# Equations

## Net July-Sept Peak (CP_kW_N_TOT[Smr])

Net July-Sept Peak is an average monthly coincident peak value.  The coincident peak is itself, the average of the loads during the five highest PG&E system loads for the month.  These average peak loads are then averaged for July through September.  “Net” refers to the peak estimate being reduced by the Net-to-Gross ratio.  This peak load estimate is derived in two ways, depending on whether hourly end use data or H-factor information is used for the end use shape.

CPKW[Smr]=CPKWN[Smr]M∗IN

Where

IN_CM,Q	=	# of cumulative units of measure M installed in year y.

For end use shapes with hourly loads:

CPKWN[Smr]M=kWh∗NTGM∗CPkWFactorS,m3

Where

kWh_A  	= 	Annual kWh reduction for measure M

NTGM	=	Net –to-Gross ratio for measure M

m	=	months 7, 8 and 9

CP_kW_FactorS,m  =   Coincident peak factor for end use shape S (corresponding to measure M) and month m.

CP_kW_FactorS,m  =  Average peak load from month m divided by total annual end use load.  The average peak load for shape S is the average end use load during the five highest PG&E system loads for the month adjusted to the chronology of the hourly end use load shapes. The residential hourly end use load shapes are from 1994, adjusted to match the chronology for 1999 by adding one day at the beginning of the shapes.   1999 is used for the base year because the hourly generation market price shape is based on 1999.  The 1994 PG&E system load indices were similarly adjusted by adding 24 to each index value.

CPKWFactorS,m=EULoadS,hr5EULoadS,hr

For end use shapes that use TOU-factors:

CPKWN[Smr]M=kWSmrM∗1∗NTGM

Where

kW_SmrM	=	Summer peak kW reduction for measure M.  For measures that have a kW demand scaler, this value is a user input.  For measures that have a kWh demand scaler, this value equals the Summer peak kW TOU factor times the annual kWh reduction for the measure.

1	=	Coincidence factor (1 by assumption)

NTGM	=	Net-to-Gross ratio for measure M

## Net Dec-Feb Peak (CP_kW_N_TOT[Wtr])

Net Dec-Feb Peak is an average monthly coincident peak value.  It is calculated the same as the Net July-Sept Peak, except that it used December, January, and February peaks, rather than July – September.

CPKW[Wtr]=CPKWN[Wtr]M∗IN

Where

IN_CM,Q	=	# of cumulative units of measure M installed in year y

For end use shapes with hourly loads:

CPKWN[Wtr]M=kWh∗NTGM∗CPkWFactorS,m3

Where

kWh_A  	= 	Annual kWh reduction for measure M

NTGM	=	Net –to-Gross ratio for measure M

m	=	months 12, 1, and 2

CP_kW_FactorS,m  =   Coincident peak factor for end use shape S (corresponding to measure M) and month m.

For end use shapes that use H-factors:

CPKWN[Wtr]M=kWWtrM∗1∗NTGM

Where

kW_WtrM	=	Winter peak kW reduction for measure M.

1	=	Coincidence factor (1 by assumption)

NTGM	=	Net-to-Gross ratio for measure M

## Noncoincident Peak (Net NCP)

For End Use shapes with hourly loads:

NCP_FM.m = Max normalized end use hourly load in month m

Normalized end use hourly loads sum to 1.0 over the 8760 observations.

Net NCP  = M [(IN_CM,y) * kWh_AM * NTGM * Maxm = 1 to 12(NCP_FM,m) ]

where

IN_CM,Q	=	# of cumulative units of measure M installed in year y

kWh_A  	= 	Annual kWh reduction for the measure

NTGM	=	Net –to-Gross ratio for measure M

NCP_FM.m	=	Noincident Peak factor for measure M in month m.

M	= 	Measure

m	=	month

S	=	End Use shape

For end use shapes that use H-factors:

NCP_FM.m = kW_SmrM,*Max (kW H-factor, for summer or winter) * NTGM

Where

Summer is April through October, Winter is the other months.

## Noncoincident Peak by Year (NNCP[t])

For End Use shapes with hourly loads:

NNCP[t]=MQ=14(foryry)INM,y∗Maxm=1to12NCP∗NTGM∗kWh

Where

INM,Q	=	# of incremental units implemented in year y for measure M

Y	=	Year of implementation

For end use shapes that use H-factors:

NNCP[t]=MQ=14(foryry)INM∗MaxbyTOUkW∗NTGM∗kWSmrMWhere

kW_H_Factor	=	H-factor by TOU period for capacity for the matching season.

kW_Smr	=	User input summer peak kW reduction for one unit.

## CEC Peak (Net CEC)

Net CEC (KW) = M [(IN_CM,y)  * kWh_AM * NTGM * (CEC_F) / 1000]

where

IN_CM,y	=	# of cumulative units of measure M installed in year yNTGM	=	Net –to-Gross ratio for measure M

CEC_F	=	0.217 (value prescribed by CEC)

## Annual Net kWh (Annual_kWh)

Annual_kWh = M Q (INM,y  * kWh_AM * NTGM * (if EULM > 1, EULM, 1))

Where

INM,y	=	# of incremental units implemented in year y for measure M.

## Lifecycle Net kWh (Lifecycle_kWh)

Lifecycle_kWh = M Q (INM,y  * kWh_AM * NTGM * LM)

Where

LM	=	 End use measure life in years.

## Annual Net Therms (Annual_Therms)

For PG&, SoCal  and SDG&E:

Annual_Thermsy = M (INM,y  * Th_AM * NTGM * (if EULM > 1, EULM, 1)

Where

Th_AM	=	Annual therm reductions for measure M

## Lifecycle Net Therms (Lifecycle_Therms)

Lifecycle_kWhy = M (INM,y  * Th_AM * NTGM * LM)

Where

LM	=	 End use measure life in years.

# Cost Effectiveness (Lifecycle Present Value Dollars)

## Cost

### TRC Cost (TOTCOSTTRC)

TOTCostTRC   = 	Program_Budget[PV]+ Net_Part_Cost[PV]

(Note that TOT_Rebate[PV] is subtracted from the TotCostTRC because TOT_Rebate[PV] is included in both Program_Budget[PV] and Net_Part_Cost[PV]).

Where

Program_Budget[PV] = 	Sum of user inputs for Administrative, Marketing/Outreach, Direct Implementation (including per measure rebate payments, upstream incentives, and direct install labor and materials), EM&V, and Performance Award

ProgramBudget[PV]=LumpSumAdmin*NPVFactor+AMQ=1Q'INM,Q∗AdminA,M1+r4Q

LumpSumAdmin  =  Program admin costs that are entered as a lump sum, rather that on a per-unit-installed basis.  Lump sum admin costs include 1) Overhead and G&A, 2) Other Admin Costs, 3) Marketing/Outreach, 4) direct implementation, and 5) User input incentive (no rebates).

NPVFactor	=	Adjustment to convert LumpSumAdmin Costs to present value dollars.  Currently set at 100% (no adjustment)

AdminA,M	=	Cost-per-unit installed admin costs for cost type A and measure M.  Cost types are 1) Incentives to entities other that the end-use customer or its assignee, 2) Direct install labor, 3) direct install materials, and 4) rebates.

Net_Part_Cost[PV]	=	Net participant cost (present value)

NetPartCost[PV]=MQ=1Q'INM,Q∗PCM∗NTGM1+r4Q

PCM	=	Participant cost for measure M. (Gross measure cost less rebates, direct install labor, direct install materials, and upstream payments)

INM,Q	=	# of incremental of measures implemented in quarter Q.

NTGM	=	Net –to-Gross ratio for measure M

Q’	=	Number of quarters of implementation.  20 for PG&E, 28 for SCE, and 12 for SDG&E and SoCalGas.

### PAC Cost (TotCostPAC)

TotCostPAC	=	Program_Budget[PV]

### RIM Cost (TotCostRIM)

TotCostRIM=TotCostPAC+My=1Q'/4BillDiscountM,y

Where

Q’	=	Number of quarters of implementation for this program.  PG&E uses a maximum of 5 yrs; SDG&E, 3 yrs; SCE, 7 years.

BillDiscountM,y=kWh∗INM,Q∗y'=1LMRateM,y'(1+r)y

Where

y	=	Year of implementation

y’	=	Year counter beginning in the year of implementation

LM	=	Measure life in years.

## Lifecycle Benefit

### Electric TRC Benefits (NetPVBenTOT[E])

NetPVBenTOT[E]=MPVBen[E]M∗NTGM

Where

NTGM	=	Net –to-Gross ratio for measure M

PVBen[E]M=Q=1Q'INM,Q∗kWh∗PV[Gen]M,Q+DmdScalerM∗PV[TD]M,Q1+r4Q

r	=	annual discount rate

Q	=	quarter, beginning in January

Q’	=	Number of quarters of implementation.  20 for PG&E, 28 for SCE, and 12 for SDG&E and SoCalGas.

INM,Q	=	# of incremental of measures implemented in quarter Q

PV[Gen]M,Q	=	Present value of avoided generation costs for measure M, corresponding to the climate zone for measure M, installed by quarter Q

DmdScaler	=	Demand Scaler.

    - Equals kWh_AM for those measures that are based on hourly shapes of use TOU demand factors that are normalized to energy (like PG&E residential).

    - Equals Summer Peak kW (input column R) for other measures.

PV[TD]M,Q	=	Present value of avoided T&D costs for measure M, corresponding to the climate zone for measure M, installed by quarter Q

For hourly end use shapes:

PV[Gen]M,Q=n=1L∗4kWEU,h∗AC[Gen]h,t,CZ(1+r4)n

L	= 	Measure life in years

h	=	hours in the quarter, Q

t	=	year corresponding to quarter Q plus n-1 years.

kWEU,h,CZ	=	Normalized load for end use EU during hour h in climate zone CZ

EU	=	End use selected to correspond to measure M

CZ	=	Climate zone specified for the measures M

AC[Gen]h,t	=	Avoided cost of generation ($/kWh) in the climate zone corresponding to the measure for hour h in year t.  This value includes environmental costs.

PV[TD]M,Q=n=1L∗4kWEU,h∗AC[TD]h,t,CZ(1+r4)n

AC[TD]h,t,CZ	=	Avoided cost of T&D ($/kWh) in the climate zone corresponding to the measure for hour h in year t.

For H-factor end use shapes.

PVBen[E]M=Q=1Q'INM,Q,CZ∗kWh∗PV[Gen]M,Q∗TOUCorrU,S+DmdScalerM∗PV[Cap]M,Q,CZ1+r4Qwhere

Q’	=	Number of quarters of implementation.  20 for PG&E, 28 for SCE, and 12 for SDG&E and SoCalGas.

PV[Gen]M,Q=n=1Trunc(L∗4,0)HF[kWh]EU,TOU∗AC[Gen]TOU,t(1+r4)n+L∗4−Trunc(L∗4,0)HF[kWh]EU,TOU∗AC[Gen]TOU,t(1+r4)nn=Trunc(L∗4,0)+1

Trunc 	=	The truncation function to round down to the nearest integer value.  The first term calculates the NPV for all whole quarters of measure life.  The second term captures the value for any remaining fractional quarters of life.

TOU	=	Time of use period corresponding to the quarter.  The three summer periods are used for quarters 2 and 3.  The two winter TOU periods are used for quarters 1 and 4.

kWM	=	Summer on-peak kW reduction for the measure.

HF[kWh]EU,TOU	=	H-factor for energy share for the EU for the TOU period

AC[Gen]TOU	=    	Simple average of implied hourly net generation avoided costs in the climate zone for the measure during the TOU period in year t.

t	=	year corresponding to quarter n.

TOUCorrU,S	=	[1 + (AC TOU Correction factor for utility u – 1) *  percent of the measure that qualifies for the correction factor].  The AC TOU correction factor varies for residential and non-residential sectors s.  If a measure has an end use that does not qualify for the AC TOU correction factor, TOUCorr is set to 1.0.  See Table 7 for AC TOU Correction factors.

PV[Cap]M,Q=n=1Trunc(L∗4,0)HF[kW]EU,TOU∗AC[TD]TOU,t(1+r4)n+(L∗4−Trunc(L∗4,0))HF[kW]EU,TOU∗AC[TD]TOU,t(1+r4)nn=Trunc(L∗4,0)+1

Trunc 	=	The truncation function to round down to the nearest integer value.  The first term calculates the NPV for all whole quarters of measure life.  The second term captures the value for any remaining fractional quarters of life

AC[TD]TOU,CZ,t  	 =  	Sum of T&D avoided costs in climate zone for the measure during the TOU period in year t.

HF[kWh]EU,TOU	=	H-factor for energy share for the EU for the TOU period

### Gas TRC Benefits (NetPVBenTOT[G])

NetPVBenTOT[G]=MPVBen[G]M∗NTGM

Where

NTGM	=	Net –to-Gross ratio for measure M

PVBen[G]M=1+r4Q=1nINM,Q∗Th∗PV[Gas]M,Q(1+r4)Q

R	=	annual discount rate

Q	=	quarter, beginning in January

n	=	Number of quarters of implementation.  20 for PG&E, 28 for SCE, and 12 for SDG&E and SoCalGas.

INM,Q	=	# of incremental of measures implemented in quarter Q

Th_AM	=	Annual gas reduction (in therms) for measure M.

PV[Gas]M,Q	=	Present value of avoided gas costs for measure M installed by quarter Q.  Expressed as $ per annual Therm reduction.  Includes procurement, environmental and T&D avoided costs.

PV[Gas]M,Q=n=1Trunc(L∗4,0)AC[GasC]Q,t+AC[GasTD]Q,t,S(1+r)n+(L∗4−Trunc(L∗4,0))AC[GasC]Q,t+AC[GasTD]Q,t,S(1+r)nn=Trunc(L∗4,0)+1

AC[GasC]Q,t	=	Gas commodity avoided cost in year t.  Cost is the average of the monthly commodity costs in the quarter.

AC[GasTD]Q,t,S	=	Gas T&D avoided cost for quarter Q in year t, based on gas seasonal usage pattern S.  T&D benefits are uniform $/Therm values for the winter months (Nov through Mar).  Therefore a gas measure that has all of its reductions in the summer would receive no T&D avoided cost value.  A measure with all of its savings in the winter would receive full TD avoided cost value.  A measure with year round savings would have the TD $/therm avoided cost reduced by a factor of 2 to reflect that fact that only half of the therm reductions (the half that occurs during the winter) would result in T&D avoided costs.

## Levelized Cost and Benefit

### Discounted Electric Reductions (kWhD[N])

kWhD[G]	=	Discounted avoided kWh – Gross.

kWhD[N]	=	Discounted avoided kWh – Net.

kWhD[G]=MkWhD[G]M

kWhD[G]M=Q=1Q'INM,Q∗kWh∗PV(r,L,−1)1+r4Q

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q.

kWh_AM  	= 	Annual kWh reduction for measure M.

Q’	=	Number of quarters of implementation.  20 for PG&E, 28 for SCE, and 12 for SDG&E and SoCalGas.

PV(r,L,-1)	= 	The uniform series present worth factor, using a discount rate of r and L number of years.

L	=	Measure life in years.

kWhD[N]M,CZ	=	kWhD[G]M,CZ * NTGM

kWhD[N]	=	M,CZ (kWhD[G]M,CZ * NTGM)

NTGM	=	Net–to-Gross ratio for measure M.

### Discounted Gas Reductions (ThD[N])

ThD[G]	=	Discounted avoided therms – Gross.

ThD[N]	=	Discounted avoided therms – Net.

ThD[G]=M,CZThD[G]M,CZ

ThD[G]M,CZ=Q=1nINM,Q,CZ∗Th∗PV(r,L,−1)1+r4Q

Th_AM	=	Annual gas reduction (in therms) for measure M.

N	=	Number of quarters of implementation

ThD[N]M,CZ	=	ThD[G]M,CZ * NTGM

NTGM	=	Net–to-Gross ratio for measure M.

### Levelized Cost

TRC Levelized Cost ($/kWh) (LC[TRC][E])

LC[TRC][E] = LB[TRC][E] – LBC[TRC][E]

Where

LB[TRC][E] 	=	Levelized benefits under the TRC test for electric (defined later)

LBC[TRC][E]	=	Levelized benefit – cost under the TRC test for electric (defined later)

PAC  Levelized Cost ($/kWh) (LC[PAC][E])

LC[PAC][E] = LB[TRC][E] – LBC[PAC][E]

Where

LB[TRC][E] 	=	Levelized benefits for electric (PAC and TRC benefits are the same)

LBC[PAC][E]	=	Levelized benefit – cost under the TRC test for electric (defined later)

RIM  Levelized Cost ($/kWh) (LC[RIM][E])

LC[RIM][E]=TotCostRIM∗NetPVBenTOT[E]NetPVBenTOT[E]+NetPVBenTOT[G]kWhD[N]

TRC Levelized Cost ($/Th) (LC[TRC][G])

LC[TRC][G] = LB[TRC][G] – LBC[TRC][G]

Where

LB[TRC][G] 	=	Levelized benefits under the TRC test for gas (defined later)

LBC[TRC][G]	=	Levelized benefit – cost under the TRC test for gas (defined later)

PAC  Levelized Cost ($/Th) (LC[PAC[G])

LC[PAC][G] = LB[TRC][G] – LBC[PAC][G]

Where

LB[TRC][G] 	=	Levelized benefits for gas  (PAC and TRC benefits are the same)

LBC[PAC][G]	=	Levelized benefit – cost under the TRC test for gas (defined later)

RIM  Levelized Cost ($/Th)  (LC[RIM][G])

LC[RIM][G]=TotCostRIM∗NetPVBenTOT[G]NetPVBenTOT[E]+NetPVBenTOT[G]ThD[N]

Where

ThD[N]	=	 Discounted lifecycle therm reductions, adjusted for the Net-to-Gross ratio.

### Levelized Benefits

TRC Levelized Benefits ($/kWh) (LB[TRC][E])

Values are the same for all cost tests.

LB[TRC][E] = NetPVBenTOT[E] / kWhD[N]

LB[PAC][E] = NetPVBenTOT[E] / kWhD[N]

LB[RIM][E] = NetPVBenTOT[E] / kWhD[N]

TRC Levelized Benefits ($/Therm) (LB[TRC][G])

Values are the same for all cost tests.

LB[TRC][G] = NetPVBenTOT[G] / ThD[N]

LB[PAC][G] = NetPVBenTOT[G] / ThD[N]

LB[RIM][G] = NetPVBenTOT[G] / ThD[N]

### Levelized Benefit – Cost

Levelized Benefits – Costs:  Electric, TRC Test, No Admin Costs. (LBC[TRC][E][NA])

LBC[TRC][E][NA]=NetPVBen[E]M−IMCPVM∗EAllocMkWhD[N]M

Where

NA	=	No Admin.  Indicates that the metric does not include admin costs.

EAllocM	=	Electric allocation for measure M.

=	NetPVBen[E]M / (NetPVBen[E]M + NetPVBen[G]M)

IMC_PV	=	Net incremental measure cost (present value).  This was defined in the Cost section.

Levelized Benefits – Costs: Gas, TRC Test, No Admin Costs. (LBC[TRC][G][NA])

LBC[TRC][G][NA]=NetPVBen[G]M−IMC∗1−EAllocMThD[N]M

Levelized Benefits – Costs:  Electric, TRC Test, with Admin Costs. (LBC[TRC][E][WA])

LBC[TRC][E][WA]=LBC[TRC][E][NA]−Admin[TRC]∗EAllocM∗MAllocMkWhD[N]M

Where

WA	=	With Admin.  Indicates that the metric does include admin costs.

Admin[TRC]	= 	Total Program Costs excluding rebates and incentives

MAllocM,CZ	=	Measure allocation.

Levelized Gas Benefits – Costs:  Gas, TRC Test, with Admin Costs. (LBC[TRC][G][WA])

LBC[TRC][G][WA]=LBC[TRC][G][NA]−Admin[TRC]∗(1−EAllocM)∗MAllocMThD[N]M

Levelized Benefits – Costs:  Electric, PAC Test, No Admin Costs. (LBC[PAC][E][NA])

LBC[PAC][E][NA]=NetPVBen[E]M−UnitIncM∗EAllocMkWhD[N]M

Where

UnitIncM,CZ	=	Present value Per unit Rebate + per unit direct install labor + per unit direct install materials.  Discounting is based on installation schedule. (see TOT_INC for discounting details)

Levelized Benefits – Costs: Gas, PAC Test, No Admin Costs. (LBC[PAC][G][NA])

LBC[PAC][G][NA]=NetPVBen[G]M−UnitIncM∗1−EAllocMThD[N]M

Levelized Benefits – Costs:  Electric, PAC Test, with Admin Costs. (LBC[RIM][E][WA])

LBC[PAC][E][WA]=LBC[PAC][E][NA]−Admin[PAC]∗EAllocM∗MAllocMkWhD[N]M

Where

Admin[PAC]	=	Total Program Costs less the sum of all UnitIncM,CZ  (this is done to avoid double counting, as those per-unit costs are included in the “no admin costs” case)

Levelized Gas Benefits – Costs:  Gas, PAC Test, with Admin Costs. (LBC[RIM][G][WA])

LBC[PAC][G][WA]=LBC[PAC][G][NA]−Admin[PAC]∗(1−EAllocM)∗MAllocMThD[N]M

# Emissions Reductions

### Electric Reductions:  CO2 tons per year (Emission[E][CO2])

Emission[E][CO2]y=Q=1+(y−1)∗4y∗4INM,Q∗kWh∗NTGM∗ER[CO2]M

Where

y	=	year of consideration. 2009 = 1.

Q	=	Quarter of the year.  Jan-Mar 2009 = 1.

INM,Q	=	# of incremental of measures implemented in quarter Q.

NTGM	=	Net–to-Gross ratio for measure M.

ER[CO2]M	=	Emission rate of CO2 in tons per kWh of measure M.

NOX and PM-10 equations are the same.  Just replace [CO2] with the appropriate indicator.  Note that CO2 emission rate is in tons per kWh.  NOX and PM-10 are in pounds per kWh.

### Gas Reductions:  CO2 tons per year (Emission[G][CO2])

Emission[G][CO2]y=Q=1+(y−1)∗4y∗4INM,Q∗Th∗NTGM∗ER[CO2]GCT

Where

y	=	year of consideration. 2009 = 1.

Q	=	Quarter of the year.  Jan-Mar 2009 = 1.

INM,Q	=	# of incremental of measures implemented in quarter Q.

NTGM	=	Net–to-Gross ratio for measure M.

ER[CO2]GCT	=	Emission rate of CO2 in tons per therm, based on the gas combustion type (GCT) specified on the input sheet for the measure.

NOX and PM-10 equations are the same.  Just replace [CO2] with the appropriate indicator.  Note that CO2 emission rate is in tons per Therm.  NOX and PM-10 are in pounds per Therm.

## Monthly Impacts (IMP_MO[X], IMP_MO[Th])

For CP, NCP, and monthly kWh.

IMPMO[X]=MkWh∗NTGM∗IMPU[X]M,m∗IN−EXP

Where

IMP_MO[X]	=	Monthly impact for metric X.  Metrics can be CP, NCP, kWh

kWh_AM  	= 	Annual kWh reduction for measure M.

NTGM	=	Net–to-Gross ratio for measure M.

IMP_U[X]M.m	=	Per unit impact for metric X.  Value is normalized to total annual kWh reductions.

IN_CM,Q	=	# of cumulative units of measure M installed by Quarter Q

PG&E:  3rd Qtr 2014

SDG&E:  3rd Qtr 2012

SCE:  4th Qtr 2011

SoCalGas:  3rd Qtr 2012

Exp_CM,Q	=	# of units of measure M, installed under this program that have reached the end of their useful life prior to Quarter Q

M	=	Measure

m	=	Month of the year (1 – 12)

For Monthly Therms

IMPMO[Th]=MkWh∗NTGM∗IMPU[Th]M,m∗IN−EXP

Where

IMP_U[Th]M,m	=	Per unit impact for therms.  Value is normalized to total annual kWh reductions, so the sum of across the 12 months = 1.0.

# Sector and End Use Shape Combinations

Table 1:  PG&E Non-Res H-Factor (TOU) based shapes.  Sector Name is at the top of the column.

Table 2:  PG&E Non-Res Hourly Load based shapes.  Sector name is at the top of the column

Table 3:  PG&E Residential Shapes

Table 4:  SCE End Use Shapes (Sectors are in Bold)

Table 5:  SDG&E End Use Measures (Not limited by sector in the spreadsheet)

Table 6:  SoCalGas Shapes

Table 7:  AC TOU Correction Factors

|  | PG&E | SCE | SDG&E | SoCalGas |
| --- | --- | --- | --- | --- |
| Residential | 1.171 | 1.202 | 1.276 | 1.202 |
| Non-Residential | 1.085 | 1.105 | 1.145 | 1.105 |