# Residential CEE RFP Calculator Tool

# Quick Guide and Equation Reference

This draft version of the model is for residential measures only.  The spreadsheet has not been copy protected, but the final user version can be ‘locked-down’ as needed.   In the final version, the user will enter data only into the INPUT tab.  The OUTPUT tabs will be shown, but locked.  The other sheets will be either locked or hidden.

This document provides a brief guide to the user inputs and outputs.  This is followed by a detailed presentation of the equations used by the Calculator Tool.  The last section presents a discussion of the data management issues and steps taken in developing the inputs for the tool.

# Inputs Summary

| Item | Location | Comment |
| --- | --- | --- |
| Proposer Name | Cell B4 | Text field |
| Program Name | Cell B5 | Text field |
| Service Territory | Cell B6 | Text field |
| Program Budget |  |  |
| Administrative Costs | E2 | Total cost over the three year life of the program. Entered in nominal dollars |
| Market/Outreach | E3 | Entered in nominal dollars |
| Direct Implementation | E4 | Entered in nominal dollars. This cell entry is for non-incentive related costs. Incentives and rebate information is entered on a per-measure basis or in cell G3. |
| Total Incentives and Rebates | E5 | Not a user entry. Sum of values in Cells G3 to G6 |
| EM&V | E6 | Entered in nominal dollars |
| Other | E7 | Entered in nominal dollars. Use this cell for items such as financing and performance award. |
| User Input Incentive | G3 | Entered in nominal dollars. Use this cell for incentives and rebates that are not calculated on a per unit basis. Users should be careful to avoid double counting incentives. If this cell is used, only enter incentive values in columns H through J for measures that are not included in this incentive calculation. |
| Direct Install Rebate | G4 | Not a user entry. Present value total based on rebates per unit entered in column H. Note that the value shown is a present value (to reflect the expected timing of the payments). PV is appropriate for cost effectiveness evaluations. Do not use this value to determine annual budget outlays. |
| Direct Install Labor | G5 | Not a user entry. Present value total based on direct install labor per unit entered in column I. Note that the value shown is a present value (to reflect the expected timing of the payments). PV is appropriate for cost effectiveness evaluations. Do not use this value to determine annual budget outlays. |
| Direct Install Materials | G6 | Not a user entry. Present value total based on direct install materials per unit entered in column J. Note that the value shown is a present value (to reflect the expected timing of the payments). PV is appropriate for cost effectiveness evaluations. Do not use this value to determine annual budget outlays. |
| Program Inputs (Beginning in Row 11) |  |  |
| Measure Name | Col A | Text field |
| Climate zone | Col B | Select from drop-down list. Cannot be left blank. Used to determine the impacts and costs for the measure. ‘System’ applies an average impact and cost shape. Note that Climate zone 3 has been subdivided into two subzones. Climate zone 3A is for San Francisco and the Peninsula. Climate Zone 3B is for the East Bay. See the attachment for a complete listing of cities and climate zones. If ‘System’ is selected, then the T&D values from the climate zone with the lowest average avoided cost (over 20 years) for the measure is used. |
| Target Sector | Col C | Leave as ‘Residential’ for the Residential measures. |
| Measure End Use Shape | Col D | Select from the drop-down list. Note that the A/C shapes vary by climate zone (denoted by the number after the colon). Be sure to match the AC shape to the Climate Zone selected in Column B. |
| Measure Type | Col E | Select from drop-down list. This entry is used to determine the Measure Life, based on values in the Program Manual. A generic program life can also be selected (at the end of the drop down list). |
| Program Type | Col F | Select from drop-down list. This entry is used to determine the Net-to-Gross ratio, based on values in the Program Manual |
| Unit Definition | Col G | Text Field. For information only. Not used in the calculations. |
| Financial Incentive: Rebate | Col H | Per unit rebates paid to the participant by the sponsoring agency. |
| Financial Incentive: Direct Install Labor | Col I | Per unit labor provided to the participant by the sponsoring agency. |
| Financial Incentive: Direct Install Materials | Col J | Per unit materials provided to the participant by the sponsoring agency. |
| Gross Incremental Measure Cost | Col K | Cost of the efficient equipment less the cost of the standard efficiency equipment. Same value as in DEER. Include initial capital costs, including sales tax, ongoing O&M costs including fuel, removal costs less salvage value, and value of customers time in arranging for installation (if significant). (PC[t]) Subtract participant avoided costs for alternate fuel devices (PA[t]) |
| Gross Unit Annual Electricity Savings | Col L | Annual net kWh reduction attributable to the installation of one unit of the measure. |
| Gross Unit Annual Gas Savings | Col M | Annual net therms reduction attributable to the installation of one unit of the measure. |
| Gas Sector | Col N | Select from the drop-down list. Used to determine the gas avoided costs to apply to the measure. (if there are gas savings) |
| Gas Savings Profile | Col P | Select from the drop-down list. Used to determine the gas avoided costs to apply to the measure. - Summer Only: All gas savings occur in April through September. - Winter Only: All gas savings occur in October through March - Annual: Gas savings occur uniformly throughout the year. |
| Effective Useful Life | Col O | Determined by the spreadsheet, based on Measure Type, or user-specified generic life. |
| Net –to-Gross Ratio | Col P | Determined by the spreadsheet, based on Program Type. |
| Installation Schedule |  |  |
| Installations by Quarter | Col T to Col AE. | Number of incremental units installed by the beginning of the quarter. For example, Q1 2006 would have all units installed prior to 1/1/2006. Q2 2006 would have all units installed from 1/1/2006 through 3/31/2006. Q3 2006 would have all units installed from 4/1/2005 through 6/30/2006. This convention of “units installed at the beginning of the quarter” results in slightly conservative estimates of benefits, as programs could be installed up to three months prior to recognition of their savings. Compared to a “middle of the quarter” convention, this approach results in present values that are 1.5% lower. As the same convention applies to both costs and benefits, the impact on net benefits or benefit cost ratios is likely negligible. |
| Comments | Col AG | For informational purposes only. Not used in the calculations. |

# Output

## Notes on the Tests

#### Total Resource Cost Test (With externalities)

| Item | Comment |
| --- | --- |
| Tax Credits (TC) | No explicit input for tax credits. |
| Levelized benefits per kWh | Use Net discounted kWh to correspond to Net benefits (NTG ratio adjusted) |

## Outputs

| Item | Location | Comment |
| --- | --- | --- |
| Total Program Budget | D4 | Total of all program budget items, including rebates and incentives |
| Incentives and Rebates | D5 | Present value of total incentives and rebates. These are already included in the Total Program Budget in D4. They are shown separately so that they can be removed from the TRC test. |
| Net Incremental Measure Cost | D6 | Present value of participant costs before any incentives or rebates, multiplied by Net-to-Gross ratio. |
| Program Impacts |  |  |
| Net Smr Pk (kW) | C9 | Sum across all measures of their average July through September coincident peaks in the fourth year of the program. Coincident peak is defined as the load during the five highest system load hours in each month. (See Equations for CP_KW). Value is adjusted for the net to gross ratio. |
| Net NCP (kW) | D9 | Sum across all measures of their individual maximum demand reductions, adjusted for net-to-gross ratios. (See Net NCP) |
| Net CEC (kW) | E9 | Sum across all measures of their “CEC” peak demand. (See Net CEC) |
| Annual Net kWh | F9 | Sum across all measures of their annual kWh savings adjusted for their net-to-gross ratio. |
| Lifecycle Net kWh | G9 | Sum across all measures of their lifecycle kWh savings, adjusted for their net-to-gross ratios. Lifecycle savings = Annual kWh * Measure Life (in yrs) * Net-to-gross ratio.* Total units installed. Note: This metric is NOT based on the units in place during the third quarter of year 4. |
| Annual Net Therms | H9 | Sum across all measures of their annual therm savings adjusted for their net-to-gross ratio. |
| Lifecycle Net Therms | I9 | Sum across all measures of their lifecycle therm savings, adjusted for their net-to-gross ratios. Lifecycle savings = Annual therms * Measure Life (in yrs) * Net-to-gross ratio.* Total units installed. Note: This metric is NOT based on the units in place during the third quarter of year 4.therms. |
| Cost Effectiveness |  | Lifecycle Present Value Dollars |
| Program TRC |  |  |
| TRC Cost | C15 | Total Program Budget + net incremental measure cost – Incentives and rebates (See TOTCostTRC) |
| TRC Benefits – Electric | D15 | Sum of net electric present value benefits for all measures installed over the three years. “Net” indicates the gross benefits are multiplied by the Net-to-Gross ratios. (See NetPVBen[E]) |
| TRC Benefits – Gas | E15 | Sum of net gas present value benefits for all measures installed over the three years. “Net” indicates the gross benefits are multiplied by the Net-to-Gross ratios. (See NetPVBen[G]) |
| NPV | G15 | TRC Benefits Electric +TRC Benefits Gas – TRC Cost |
| BC Ratio | H15 | (TRC Benefits Electric + TRC Benefits Gas) / TRC Cost. Note that the B/C Ratio is an approximation because any costs associated with increased usage are treated as negative benefits rather than as a cost as in the Standard Practice Manual |
| PAC (Program Administrator Cost Test) |  |  |
| PAC Cost | C16 | Total program budget, including customer incentive payments. (See TOTCostPAC) |
| PAC Benefits – Electric | D16 | Same as TRC test. Note that the benefits include avoided environmental externalities. |
| PAC Benefits – Gas | E16 | Same as TRC test |
| Incentives | F16 | NA. Already included in PAC Cost above |
| PAC NPV | G16 | PAC Benefits Electric +PAC Benefits Gas – PAC Cost |
| PAC BC Ratio | H16 | (PAC Benefits Electric + PAC Benefits Gas) / PAC Cost. Note that the B/C Ratio is an approximation because any costs associated with increased usage are treated as negative benefits rather than as a cost as in the Standard Practice Manual |
| Levelized Costs and Benefits |  |  |
| TRC Discounted kWh | C24 | Discounted lifecycle avoided kWh, adjusted for Net-to-gross ratio for each measure. (See kWhD[N]) |
| PAC Discounted kWh | C25 | Same as TRC Discounted kWh. |
| TRC Discounted Therms | D27 | Discounted lifecycle avoided therms, adjusted for Net-to-gross ratio for each measure. (See ThD[N]) |
| PAC Discounted Therms | D28 | Same as TRC Discounted Therms. |
| TRC Cost | E24, E27 | (TRC Benefits) less the (TRC Benefits – Costs) |
| PAC Cost | E25, E28 | (PAC Benefits) less the (PAC Benefits – Costs) |
| TRC Benefits | F24, F27 | TRC benefits (by fuel type) divided by the TRC discounted kWh or Therms. |
| PAC Benefits | F25, F28 | PAC benefits (by fuel type) divided by the PAC discounted kWh or Therms. |
| TRC Benefits - Costs | G24, G27 | Sum over all measures of TRC benefits less the allocated TRC costs for each measure. For each measure, unit costs (net IMC) are allocated between electric and gas in proportion to the benefits by fuel for that measure. The remaining program costs are allocated to each measure based on each measure’s share of the total electric and gas benefits. Once allocated to each measure, the remaining program costs are allocated to electric and gas in proportion to the benefits by fuel for that measure. |
| PAC Benefits - Costs | G25, G28 | Sum over all measures of PAC benefits less the allocated PAC costs for each measure. For each measure, unit costs (per unit rebates, direct install labor, and direct install materials) are allocated between electric and gas in proportion to the benefits by fuel for that measure. The remaining program costs are allocated to each measure based on each measure’s share of the total electric and gas benefits. Once allocated to each measure, the remaining program costs are allocated to electric and gas in proportion to the benefits by fuel for that measure. |
| Net Impacts Summary by Year |  |  |
| Net Smr Peak | C35:C38 | Incremental net average summer peak reductions for measures installed since the 4th quarter of the prior year and until the 3rd quarter of the year of concern. The summer peak impacts are based on the average coincident impact per measure from July through September, adjusted for the Net –to-gross ratio. Note that the Net Smr Peak reduction for programs installed for the 4th quarter of 2008 are shown in the row for 2009. |
| Net NCP | D35:D37 | Maximum monthly NCP for the year. (See NNCP[t]). Note that the sum of these annual Net NCP values will likely be lower than the Net NCP value shown in the Program Inputs section. As some measures will have maximum reductions in different months, the NCP reductions in any particular month will be lower than the annual NCP reduction (which uses the maximum reduction for each measure, regardless of when that reduction occurs) |
| Net CEC | E35:E37 | Total program CEC Peak allocated to years based on the annual kWh for programs installed in the year of concern. |
| Annual Net kWh | F35:F37 | # of measures installed in the year * Annual kWh reduction per measure * Net-to-gross ratio |
| Lifecycle Net kWh | G35:G37 | Total program lifecycle net kWh reductions allocated to years based on the annual kWh for programs installed in the year of concern. |
| Annual Net Therms | H35:H37 | Total program net therm reductions allocated to years based on the annual kWh for programs installed in the year of concern. |
| Lifecycle Net Therms | I35:I37 | Total program lifecycle net therm reductions allocated to years based on the annual kWh for programs installed in the year of concern. |
| Net Impacts by Sector |  | Not applicable for the residential sheet, as all measures are for the residential sector |
| Net Impacts by Climate Zone |  | Same information as the Impacts Summary section, except segmented by location instead of year of installation. CP information is for units in place in 3rd quarter of 4th year. |
| Monthly Impacts |  | All impacts are based on the devices that would be in place in that month. All measures are assumed to be installed on the first say of the quarter (no ramp-up). (see IMP_MO[x]). Note that the NCP values will not match what is shown in the Program Input section As some measures will have maximum reductions in different months, the NCP reductions in any particular month will be lower than the annual NCP reduction (which uses the maximum reduction for each measure, regardless of when that reduction occurs) |

## Output- Measure

| Net Impacts by Measure |  | Same information as the Impacts Summary section, except segmented by measure instead of year of installation. CP information is for units in place in 3rd quarter of 4th year. |
| --- | --- | --- |
| Levelized Benefits less Costs |  |  |
| w/ allocated Admin | Cols K,L,O,P | Same information as in the Output Tab for levelized results, but reported by measure. |
| No Allocated Admin – TRC | Cols I,J | Only includes net IMC in costs. Program costs and other admin costs are not included. |
| No Allocated Admin - PAC | Cols M,N | Includes per unit costs for 1) rebates, 2) direct installation, and 3) direct install materials. Other rebates and incentives (entered as a lump sum on the Input tab), as well as other program and admin costs are not included. |

# Equations

## Net Avg Summer Peak (CP_kW_N)

Average Summer Peak is an average monthly coincident peak value.  The coincident peak is itself, the average of the loads during the five highest PG&E system loads for the month.  These average peak loads are then averaged for July through September to arrive at the Average Summer Peak.  “Net” refers to the peak estimate being reduced by the Net-to-Gross ratio.

CPKW=kWh∗NTGM∗CPkWFactorS,m

Where

kWh_A  	= 	Annual kWh reduction for measure M

NTGM	=	Net–to-Gross ratio for measure M

CP_kW_FactorS,m  =   Coincident peak factor for end use shape S (corresponding to measure M) and month m.  This factor is derived in two ways, depending on whether hourly end use data or H-factor information is used for the end use shape.

For end use shapes with hourly loads:

CP_kW_FactorS,m  =  Average peak load form month m divided by total annual end use load.  The average peak load for shape S is the average end use load during the five highest PG&E system loads for the month adjusted to the chronology of the hourly end use load shapes. The residential hourly end use load shapes are from 1994, adjusted to match the chronology for 1999 by adding one day at the beginning of the shapes.   1999 is used for the base year because the hourly generation market price shape is based on 1999.  The 1994 PG&E system load indices were similarly adjusted by adding 24 to each index value.

CPKWFactorS,m=EULoadS,hr5EULoadS,hr

For end use shapes with H-factors:

CP_kW_FactorS,m	=  	HF_PS for Smr On-peak / 786 hours (Apr-Sept); or
HF_PS for Wtr-Ptl / 1638 hours (Oct – Mar)

where

HF_PS	=	H-Factor (Peak) for shape S

M	= 	Measure

m	=	month

S	=	End Use shape

## Noncoincident Peak

Net NCP  = M IN_CM,Q * kWh_AM * NTGM * Maxm = 1 to 12(NCP_FM,m)

where

IN_CM,Q	=	# of cumulative measures installed by Quarter Q

kWh_A  	= 	Annual kWh reduction for the measure

NTGM	=	Net –to-Gross ratio for measure M

NCP_FM.m	=	Noincident Peak factor for measure M in month m.

M	= 	Measure

m	=	month

For End Use shapes with hourly loads:

NCP_FM.m = Max normalized end use hourly load in month m

Normalized end use hourly loads sum to 1.0 over the 8760 observations.

For end use shapes with H-factors:

NCP_FM.m = CP_KWM,S,m

## Noncoincident Peak by Year

NNCP[t]=MCZQ=14(foryrt)INM,Q,CZ∗Maxm=1to12NCP∗NTGM∗kWh

Where

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q in climate zone CZ.

T	=	Year of implementation

## CEC Peak

Net CEC (KW) = M IN_CM,Q * kWh_AM * NTGM * (CEC_F) / 1000

where

IN_CM,Q	=	# of cumulative measures installed, and still in place in quarter Q

kWh_A  	= 	Annual kWh reduction for the measure

NTGM	=	Net –to-Gross ratio for measure M

CEC_F	=	0.217 (value prescribed by CEC)

## Electric TRC Benefits

NetPVBen[E]M,CZ = PVBen[E]M,CZ * NTGM

Where

NTGM	=	Net –to-Gross ratio for measure M

CZ	=	Climate zone

PVBen[E]M,CZ=(1+r)Q=112INM,Q,CZ∗kWh∗PV[Gen]M,Q+PV[TD]M,Q,CZ1+r4Q

R	=	annual discount rate

Q	=	quarter, beginning in January

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q in climate zone CZ.

PV[Gen]M,Q	=	Present value of avoided generation costs for measure M installed by quarter Q

PV[TD]M,Q,CZ	=	Present value of avoided T&D costs for measure M installed by quarter Q in climate zone CZ.

For hourly end use shapes:

PV[Gen]M,Q,CZ=n=1LkWEU,h,CZ∗AC[Gen]h,t(1+r)n

L	= 	Measure life in years

h	=	hours in the quarter, Q

t	=	year corresponding to quarter Q plus n-1 years.

kWEU,h,CZ	=	Normalized load for end use EU during hour h

EU	=	End use selected to correspond to measure M

CZ	=	Climate zone

AC[Gen]h	=	Avoided cost of generation ($/kWh) for hour h in year t.  This value includes environmental costs.

PV[TD]M,Q,CZ=n=1LkWEU,h,CZ∗AC[TD]h,t,CZ(1+r)n

AC[TD]h,t,CZ	=	Avoided cost of T&D ($/kWh) for hour h in year t in climate zone CZ

For H-factor end use shapes.  Note that for H-factor-based shapes, avoided generation costs are calculated as using both an energy and a capacity piece.  The capacity piece is the all-in market value in excess of the cost of operating a simple cycle gas turbine.  The energy cost is the market value below the simple cycle gas turbine variable cost.

PV[Gen]M,Q,CZ=n=1LHF[kWh]EU,TOU∗AC[NetGen]TOU,t(1+r)n+n=1LHF[kW]EU,TOU∗AC[GenCap]TOU,t(1+r)n

TOU	=	Time of use period corresponding to the quarter.  The three summer periods are used for quarters 2 and 3.  The two winter TOU periods are used for quarters 1 and 4.

HF[kWh]EU,TOU	=	Energy (kWh) H-factor.

AC[NetGen]TOU,t	=    Simple average of hourly net generation avoided costs during the TOU period in year t.  “Net” refers to the energy avoided costs net of the value of generation capacity.

HF[kW]EU,TOU	=	Demand (kW) H-factor.

AC [GenCap]TOU,t	=    Sum of hourly generation avoided capacity costs during the TOU period in year t.

HrsTOU	=	Number of hours in the TOU period.

t	=	year corresponding to quarter Q plus n-1 years.

PV[TD]M,Q,CZ=n=1LHF[kW]EU,TOUHrsTOU∗AC[TD]TOU,CZ,t(1+r)n

AC [TD]TOU,CZ,t   =    Sum of hourly T&D avoided costs for climate zone CZ during the TOU period in year t.

### Gas TRC Benefits

NetPVBen[G]M,CZ = PVBen[G]M,CZ * NTGM

Where

NTGM	=	Net –to-Gross ratio for measure M

CZ	=	Climate zone.  Not needed for gas, but measures with both electric and gas savings may be entered by climate zone to better estimate electric avoided costs.

PVBen[G]M,CZ=(1+r)Q=112INM,Q,CZ∗Th∗PV[Gas]M,Q(1+r4)Q

R	=	annual discount rate

Q	=	quarter, beginning in January

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q in climate zone CZ.

Th_AM	=	Annual gas reduction (in therms) for measure M.

PV[Gas]M,Q	=	Present value of avoided gas costs for measure M installed by quarter Q.  Expressed as $ per annual Therm reduction.  Includes procurement, environmental and T&D avoided costs.

PV[Gas]M,Q=n=1LAC[GasC]Q,t+AC[GasTD]Q,t,S(1+r)n

AC[GasC]Q,t	=	Gas commodity avoided cost in year t.  Cost is the average of the monthly commodity costs in the quarter.

AC[GasTD]Q,t,S	=	Gas T&D avoided cost for quarter Q in year t, based on gas seasonal usage pattern S.  T&D benefits are uniform $/Therm values for the winter months (Nov through Mar).  Therefore a gas measure that has all of its reductions in the summer would receive no T&D avoided cost value.  A measure with all of its savings in the winter would receive full TD avoided cost value.  A measure with year round savings would have the TD $/therm avoided cost reduced by a factor of 2 to reflect that fact that only half of the therm reductions (the half that occurs during the winter) would result in T&D avoided costs.

### Costs

TOTCostTRC   = 	Total Program Budget + IMC_N – TOT_INC

Where

Program Budget 	= 	Sum of user inputs for Administrative, Marketing/Outreach, Direct Implementation (including per measure rebate and incentive payments), EM&V, and Performance Award

IMC_N	=	Net incremental measure cost (present value)

IMCN=M,CZ1+rQ=112INM,Q,CZ∗PCM,CZ∗NTGM1+r4Q

PCM,CZ	=	Incremental cost for Measure M in climate zone CZ.

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q in climate zone CZ.

NTGM	=	Net –to-Gross ratio for measure Ma

TOT_INC	=	Sum of rebate and incentive payments provided to participants

TOTINC=INC+M,CZ1+rQ=112INM,Q,CZ∗RebateM,CZ+DI+DI1+r4Q

INC	=	User input value for lump sum incentive costs

RebateM,CZ	=	Rebate costs for Measure M in climate zone CZ.

DI_LM,CZ	=	Direct install labor incentive costs for Measure M in climate zone CZ.

DI_MM,CZ	=	Direct install materials incentive costs for Measure M in climate zone CZ.

TOTCostPAC = Program_Budget

where

Program Budget 	= 	Sum of user inputs for Administrative, Marketing/Outreach, Direct Implementation, Rebates and Incentives, EM&V, and Performance Award.

### Discounted Energy

kWhD[G]	=	Discounted avoided kWh – Gross.

kWhD[N]	=	Discounted avoided kWh – Net.

kWhD[G]=M,CZkWhD[G]M,CZ

kWhD[G]M,CZ=1+rQ=112INM,Q,CZ∗kWh∗PV(r,L,−1)1+r4Q

INM,Q,CZ	=	# of incremental of measures implemented in quarter Q in climate zone CZ.

kWh_AM  	= 	Annual kWh reduction for measure M.

PV(r,L,-1)	= 	The uniform series present worth factor, using a discount rate of r and L number of years.

L	=	Measure life in years.

kWhD[N]M,CZ	=	kWhD[G]M,CZ * NTGM

kWhD[N]	=	M,CZ (kWhD[G]M,CZ * NTGM)

NTGM	=	Net–to-Gross ratio for measure M.

ThD[G]	=	Discounted avoided therms – Gross.

ThD[N]	=	Discounted avoided therms – Net.

ThD[G]=M,CZThD[G]M,CZ

ThD[G]M,CZ=1+rQ=112INM,Q,CZ∗Th∗PV(r,L,−1)1+r4Q

Th_AM	=	Annual gas reduction (in therms) for measure M.

ThD[N]M,CZ	=	ThD[G]M,CZ * NTGM

NTGM	=	Net–to-Gross ratio for measure M.

## Levelized Benefits – Costs

Levelized Benefits – Costs:  Electric, TRC Test, No Admin Costs.

LBC[TRC][E][NA]M,CZ=NetPVBen[E]M,CZ−IMC∗EAllocM,CZkWhD[N]M,CZ

Levelized Benefits – Costs: Gas, TRC Test, No Admin Costs.

LBC[TRC][G][NA]M,CZ=NetPVBen[G]M,CZ−IMC∗1−EAllocM,CZThD[N]M,CZ

Levelized Benefits – Costs:  Electric, TRC Test, with Admin Costs.

LBC[TRC][E][WA]M,CZ=LBC[TRC][E][NA]M,CZ−Admin[TRC]∗EAllocM,CZ∗MAllocM,CZkWhD[N]M,CZ

Levelized Gas Benefits – Costs:  Gas, TRC Test, with Admin Costs.

LBC[TRC][G][WA]M,CZ=LBC[TRC][G][NA]M,CZ−Admin[TRC]∗(1−EAllocM,CZ)∗MAllocM,CZThD[N]M,CZ

Levelized Benefits – Costs:  Electric, PAC Test, No Admin Costs.

LBC[PAC][E][NA]M,CZ=NetPVBen[E]M,CZ−UnitIncM,CZ∗EAllocM,CZkWhD[N]M,CZ

Levelized Benefits – Costs: Gas, PAC Test, No Admin Costs.

LBC[PAC][G][NA]M,CZ=NetPVBen[G]M,CZ−UnitIncM,CZ∗1−EAllocM,CZThD[N]M,CZ

Levelized Benefits – Costs:  Electric, PAC Test, with Admin Costs.

LBC[PAC][E][WA]M,CZ=LBC[PAC][E][NA]M,CZ−Admin[PAC]∗EAllocM,CZ∗MAllocM,CZkWhD[N]M,CZ

Levelized Gas Benefits – Costs:  Gas, PAC Test, with Admin Costs.

LBC[PAC][G][WA]M,CZ=LBC[PAC][G][NA]M,CZ−Admin[PAC]∗(1−EAllocM,CZ)∗MAllocM,CZThD[N]M,CZ

Where

EAllocM,CZ	=	Electric allocation for measure M in climate zone CZ.

=	NetPVBen[E]M,CZ / (NetPVBen[E]M,CZ + NetPVBen[G]M,CZ)

MAllocM,CZ	=	Measure allocation.

UnitIncM,CZ	=	Present value Per unit Rebate + per unit direct install labor + per unit direct install materials.  Discounting is based on installation schedule. (see TOT_INC for discounting details)

Admin[TRC]	= 	Total Program Costs excluding rebates and incentives

Admin[PAC]	=	Total Program Costs less the sum of all UnitIncM,CZ  (this is done to avoid double counting, as those per-unit costs are included in the “no admin costs” case)

### Monthly Impacts

For CP, NCP, and monthly kWh.

IMPMO[X]=MkWh∗NTGM∗IMPU[X]M,m∗IN

Where

IMP_MO[X]	=	Monthly impact for metric X.  Metrics can be CP, NCP, kWh

kWh_AM  	= 	Annual kWh reduction for measure M.

NTGM	=	Net–to-Gross ratio for measure M.

IMP_U[X]M.m	=	Per unit impact for metric X.  Value is normalized to total annual kWh reductions.

IN_CM,Q	=	# of cumulative measures installed, and still in place in quarter Q

M	=	Measure

m	=	Month of the year (1 – 12)

For Monthly Therms

IMPMO[Th]=MkWh∗NTGM∗IMPU[Th]M,m∗IN

Where

IMP_U[Th]M,m	=	Per unit impact for therms.  Value is normalized to total annual kWh reductions, so the sum of across the 12 months = 1.0.

# Load Shape Mapping

With the development of the hourly avoided costs by climate zone, the benefits of an efficiency measure depends on the timing of the customer reductions.  Ray Wong and his staff produced hourly estimates of efficiency impacts for each CEC climate zone that PG&E serves which are used in the analysis of the benefits of each measure.  This section describes which load shapes are provided, how they are organized, and used in developing inputs for the tool.

Residential Measures

The residential load shapes come from the 1994 residential AMP survey data.  Therefore, the hourly loads correspond to the weather and order of days from 1994.

In total, 18 sets of 8760 load shapes were provided.  These span 13 enduses, with air conditioning provided for multiple climate zones.  In combination with the 9 climate zones, the 13 enduses make a total of 117 sets of avoided cost estimates.

Residential A/C 1994 AMP

Air Conditioning was provided for each of the climate zones.  Because of limited survey data availability and similarity of the climate zones, a single shape was provided for multiple areas.  The following list was provided;

CZs: 4, 11, 12, 13, (2,16), (3,5) : total of 6 sets of 8760 data

Residential non-A/C enduses 1994 AMP

12 Enduses were provided that were averaged across the entire PG&E territory.  Note that some of the enduses are weather dependent (window A/C, heat pump compressor hourly load) – ideally these would be available by climate zone, but the sample size is not large enough to span all of the climate zones and maintain enough observations.  Instead, these hourly shapes are averaged across the entire service territory.

| Electric End use Load Shape | Label in Spreadsheet | Survey Sample Size |
| --- | --- | --- |
| CLOTHES DRYER HOURLY LOAD (KWH) | DRY | 453 |
| FREEZER HOURLY LOAD (KWH) | FRZ | 74 |
| HEAT PUMP COMPRESSOR HOURLY LOAD (KWH) | HEAT_PUMP | 27 |
| MICROWAVE HOURLY LOAD (KWH) | MICRO | 48 |
| POOL PUMP HOURLY LOAD (KW) | PPPHRYLY | 10 |
| REFRIGERATOR HOURLY LOAD (KW) | REFRIG | 270 |
| STOVE/OVEN HOURLY LOAD (KW) | STOVE&OVEN | 190 |
| SPA/HOT TUB/SPA HEATER HOURLY LOAD (KW) | SPA | 10 |
| STOVE ONLY HOURLY LOAD (KW) | STOVE | 124 |
| WINDOW AIR CONDITIONER HOURLY LOAD (KW) | WINDOW AC | 41 |
| WATER HEATER HOURLY LOAD (KW) | DHW | 117 |
| CLOTHES WASHER HOURLY LOAD (KW) | CLOTHES | 49 |

## Converting Data order to 1991

### Residential

Mapping 1994 year residential shapes to 1991 order to line up with the avoided costs:

1/1/94 is Saturday.  1/1/1991 is Tuesday.  Converted to 1991 by removing 1st three days of hourly data set (hours 1 through 72).

Last day of 1991 is a Tuesday, last day of 1994 is Saturday.  Fill out the missing days by adding the second and third days (that were removed) after last day in data set.  Data is then normalized so that each end use sums to 1.0.

## Use of Hourly Data

### Coincident Peak

Five hour average load for each month.  The five hours selected are those that had the highest PG&E system loads in 1994.  The average is performed using index values to select the 60 (5 hours * 12 months) values from the 8760 end use shape.  The index values were adjusted to reflect the 1999 order mapping discussed above.

### Generation Value

Generation Value is the sumproduct of the hourly generation avoided costs and the hourly end use load shape.  The generation avoided costs reflect the market shape in 1999.  The end use shapes reflect the 1994 consumption, adjusted for day of the week chronology.  The estimate of generation value is conservative to the extent that high usage days do not correspond to high price days.

### T&D Value

T&D Value is the sumproduct of the hourly T&D avoided costs and the hourly end use shapes.  The T&D shapes reflect the peak capacity allocation factors based on TMY weather files.  TMY weather is representative, and does not correspond to any particular year.  Furthermore, weather is independent of day of the week issues, so no adjustment to chronology of the end use shapes is required.  The CEC is in the process of updating its DEER database using the same TMY datafiles as used in the avoided cost model.  Until that update is completed, the T&D Value estimates are conservative to the extent that high usage hours do not correspond to high cost hours.

## H-Factors

For enduses that do not have enduse load shapes we have h-factor data that can be used as a proxy.  PG&E provided h-factors for 45 enduse shapes.  The shapes are not differentiated by climate zone, so each was used to calculate avoided costs for each of the 9 climate zones.  This results in a total of 405 combinations of avoided cost estimates.

Generation Value

Average avoided net energy cost by TOU period is combined with the H-factor TOU consumption shares.  In addition, the H-factor demand by TOU period information is combined with the generation capacity costs by TOU period.

T&D Value

H-factor demand by TOU period information is combined with the T&D capacity costs by TOU period.

H-Factor Shapes

20 = Res. Spc. Cooling

21 = Res. Wtr. Heating

22 = Res. Ht. Pump Cooling

23 = Res. Ele. & Ht. Pump Heating

24 = Res. Refrigeration

25 = Res. Lighting

26 = Res. Central Air Conditioning

27 = Removed

28 = Res. New Const. Lighting

29 = Res. New Const. Cooling

30 = Res. A/C Downsize

31 = Res. Cooking

32 = Res. Clothes Dry

33 = Res. Insul. Cen. A/C

34 = Res. Insul. Elect. Heat

35 = Res. Ceil. Insul. HP Cooling

36 = Res. Ceil. Insul. HP Heating

37 = Res. Wall Insul. HP Cooling

38 = Res. Wall Insul. HP Heating

39 = Res. Flr. Insul. HP Cooling

40 = Res. Flr. Insul. HP Heating

41 = Res. EMS

42 = Res. Dir. Assist. Weatherization

43 = Res. Dir. Assist. Refrigeration

44 = Res. Dir. Assist. Lighting

45 = Res. Dir. Assist. Evap. Cooler