# E3 Calculator.  Version 4 changes

## Changes to the calculation of TRC Costs

Definitions

Admin$	=	Administrative costs excluding all incentives

Meas$		=	Gross or Incremental Measure Cost

Rebate$	=	Rebate to end use customer or its assignee

OtherInc$	=	Incentives to parties other than the customer or its assignee

DirectI$	=	Direct install labor and materials

Prior E3 Calculator (Versions 3c7 and prior)

TRC Cost	=	E4+E6-E5

E4	=	Admin$ + Rebate$ + OtherInc$ + DirectI$

E6	=	NTG * (Meas$ - OtherInc$ - DirectI$)

E5	=	Rebate$

So,

TRC Cost 	=	Admin$ + NTG*Meas$ + OtherInc$ + DirectI$  + NTG * (-

Upstream$ - DirectI$)

=	Admin$ + NTG*Meas$ + (1-NTG) * (OtherInc $ + DirectI$)

Revision per Decision 07-09-043

http://www.cpuc.ca.gov/PUBLISHED/FINAL_DECISION/73172.htm

TRC Cost 	=	Admin$ + NTG*Meas$ + (1-NTG) * (Rebate$ + OtherInc$ + DirectI$)

To implement the change, the following changes were made to the E3 Calculators

TRC Cost	=	E4 + E6

E4	=	Admin$ + Rebate$ + OtherInc$ + DirectI$

E6	=	NTG * (Meas$ - Rebate$ - OtherInc$ - DirectI$)

“Participant Cost” on the 3c version of the input sheets was defined as: (Meas$ - OtherInc$ - DirectI$).  In version 4, this has been changed to: (Meas$ - Rebate$ - OtherInc$ - DirectI$).  The title of column Q of the input tab was changed accordingly from “Participant Cost (prior to rebates)” to “Participant Cost.”  Other changes were then made throughout the calculation tab to account for rebate costs being subtracted from the participant cost.

## Step by Step Revisions:

Input Tab

Change Input Col Q (participant cost) to subtract rebates

Change formatting of Col Q (=L17-M17-N17-O17-P17)

Change Col Q label to “Participant Cost ($/unit)”

Output Tab

Change Output TRC Calculation from E4 + E6 – E5 to E4 + E6

Change label of B6 to Net Participant Cost ($)

Change D6 to a Net calculation by adding NTG term

=SUMPRODUCT(Input!Q17:Q316,Input!AF17:AF316,Input!BR17:BR316)

Remove Rebate from Col 5

Add Nominal and Present  Value labels to Row 1

Calculations Tab

Change GA label to: “Gross Participant Cost PV ($)”

Change GB label to:  “Net Participant Cost PV ($)”

Add rebates to calculations (since no longer in the participant cost)

Change Col GI: TRC Net Benefit  (Formula shown for the cell in row 17)

=ES17+ET17-(GB17+FW17 +FX17+FY17+FZ17)-(Input!$I$13-Input!$G$11)*GH17

Change Col GJ:  TRC Electric Net Benefit (no admin)

=IF(GE17<>0,(ES17-(GB17+FW17 +FX17+FY17+FZ17)*GG17)/GE17,0)

Change Col GK:  TRC Gas Net Benefit (no admin)

=IF(GF17<>0,(ET17-(GB17+FW17 +FX17+FY17+FZ17)*(1-GG17))/GF17,0)

Col GR:  TRC B/C Ratio – no admin

=(ET17+ES17)/(GB17+FW17+FX17+FY17+FZ17)

Col GT:  TRC B/C Ratio with admin

=(ET17+ES17)/((GB17+FW17 +FX17+FY17+FZ17)+$GH17*(Input!$I$13-Input!$G$11))

## Other Changes

PG&E Input Tab:  Unshade input column AD

Change NTG lookups for 2 entries, replicate erroneous entry but correct the value it refers to.

Large Standard Performance Contract 0.7011 (Change to 0.70)

Savings by Design 0.8212 (Change to 0.82)

Modify Macro Import Data routine

SoCal Gas

Change Call CopyPasteData(FileName, MasterFileName, "U", "AC") to

Call CopyPasteData(FileName, MasterFileName, "U", "AE")  	'Revised AC to AE, 10/9/07

Call CopyPasteData(FileName, MasterFileName, "AG", "AG")    	'Added 10/9/07

PG&E

Call CopyPasteData(FileName, MasterFileName, "U", "AD")     	'Revised 10/9/07 BKH

SCE

Call CopyPasteData(FileName, MasterFileName, "U", "AD")     	'Revised 10/9/07 BKH